Bhai, this looks like a comprehensive final lab assessment or a CompTIA N+ level exam paper! Let's break down all these sections into simple, structured, and technically accurate answers that an average college student can easily write to score top marks.

Section A: Theoretical Framework

1. Hardware Selection and Bottleneck Prevention

To support a startup team doing concurrent development (compiling code, Docker, IDEs) and video conferencing (Zoom/Teams running simultaneously), hardware choices must prevent resource starvation:

- CPU (The Engine): Choose processors with high core and thread counts (Minimum 8 Cores / 16 Threads, e.g., AMD Ryzen 7 or Intel Core i7). Video rendering/conferencing uses parallel processing, while code compilation scales heavily with more cores.

- RAM (The Workspace): Minimum 32 GB DDR4/DDR5. Development environments (like VS Code or Android Studio) and web browsers eat massive amounts of RAM. Anything less than 32GB will force the OS to use virtual memory (paging), grinding the system to a halt.

- Storage (The Highway): NVMe M.2 SSD is mandatory for the primary drive. Standard HDDs or even SATA SSDs create a severe bottleneck when the CPU has to wait for large project files or source code to read/write.

- Compatibility Check: Ensure the Motherboard chipset natively supports the CPU socket type, and the motherboard's QVL (Qualified Vendors List) officially lists the selected RAM model and speed.

2. Resource Allocation & Background Apps Bottleneck

When intensive background applications like corporate Antivirus (AV) software are deployed, they cause performance degradation through Resource Contention:

- CPU Exhaustion: Antivirus software runs continuous real-time scanning. Every time a developer compiles code or modifies a file, the AV intercepts the operation to scan it. This creates a high CPU interrupt cycle queue, causing micro-stutters.

- Disk I/O Bottleneck: AV software heavily utilizes disk read/write bandwidth. If the machine uses a mechanical HDD or a slow SATA SSD, the disk active time hits 100%, preventing active foreground development tools from fetching data.

- RAM Allocation: If background software consumes a fixed chunk of primary memory, the operating system is forced to move background tabs into Swap Space/Pagefile, slowing down execution speeds dramatically.

3. Systematic Troubleshooting for Inconsistent Peripheral Failures

When a peripheral (e.g., a high-end external capture card or specialized USB device) works on Workstation A but fails or acts inconsistently on Workstation B, follow this systematic isolation process:

- Isolate the Hardware: Test the device using a different, certified high-speed cable on a native motherboard USB port (avoid external unpowered hubs).

- Verify Device Manager Status: Check Windows devmgmt.msc. Look for a yellow triangle exclamation mark indicating a driver conflict or error code (like Code 10 or Code 43).

- Audit Driver Version: Compare working vs. non-working machines. Often, the operating system automatically installs a generic, unstable Microsoft driver via Windows Update instead of the manufacturer's proprietary driver.

- Clean Reinstallation: Use the Roll Back Driver feature, or completely uninstall the device, disconnect it from the network (to block automatic updates), and install the specific OEM vendor driver manually.

- Check OS Architecture/Build: Ensure both machines are running the exact same Windows/Linux patch build version, as operating system kernel updates can occasionally break backward compatibility for older device drivers.

4. Role of UPS and Surge Protection

- Surge Protection: Blocks sudden high-voltage spikes (caused by lightning strikes or power grid switching) using MOVs (Metal Oxide Varistors) that divert excess electricity to the ground, protecting delicate silicon computer microchips from physically burning out.

- UPS (Uninterruptible Power Supply): Provides instant, battery-backed power when the main grid fails.

- Preventing File System Corruption: When an OS writes data to storage, it keeps metadata changes inside volatile RAM cache before finalizing them on disk. A sudden power cut stops this process halfway, leading to an incomplete write (torn write). The next time the PC boots, the file system structure (like the NTFS Master File Table or ext4 journal) becomes unreadable. A UPS gives the system enough time to gracefully flush the cache and execute a safe shutdown sequence.

Section B: Practical Application

Task 1: Physical Assembly Sequence

When assembling a desktop workstation in a hardware lab, always use this logical sequence to avoid space constraints inside the chassis:

- Preparation: Place the motherboard on an anti-static mat and install the CPU into the socket, aligning the triangle markers.

- RAM & Storage: Insert the RAM modules into slots A2/B2 (for dual channel) and secure any M.2 NVMe SSDs into their motherboard slots.

- Cooler: Apply thermal paste (if not pre-applied) and mount the CPU Cooler/Heatsink over the processor. Connect its fan wire to the CPU_FAN header.

- Case Preparation: Install the motherboard I/O shield into the back of the case cabinet, and screw in the motherboard standoffs.

- Mounting: Lower the assembled motherboard gently into the case and secure it using screws.

- Power Supply (PSU): Mount the PSU in the cabinet basement. Route and plug in the massive 24-pin ATX Main Power cable and the 8-pin CPU Power cable.

- Front Panel & Peripherals: Connect the case front panel wires (Power button, Reset button, HDD LED, USB, and Audio headers).

- GPU: Insert the dedicated Graphics Card into the top PCI-E x16 slot and attach its external PCIe power cables.

Task 2: Troubleshooting a RAM Mismatch Error

If a computer throws a RAM mismatch error, fails to boot at rated speeds, or experiences stability boot-loops, fix it within BIOS/UEFI using these exact troubleshooting steps:

- Turn on the PC and repeatedly tap Delete or F2 to enter the BIOS/UEFI setup utility.

- Navigate to the Advanced Memory Settings or Overclocking tab (often named Ai Tweaker, Extreme Tweaker, or MIT).

- Locate the RAM profile configuration. If XMP (Intel) or EXPO/DOCP (AMD) is enabled, disable it and revert the memory profile back to standard Auto or baseline JEDEC timings.

- Manually force the memory frequency to match the clock speed of the slowest installed RAM stick (e.g., if you paired a 3200MHz stick with a 2600MHz stick, lock the speed down to 2600MHz).

- Slightly increase the RAM voltage manually to standard stable values (like 1.35V) if the mixed modules are struggling to hold timings together.

- Press F10 to save the configured adjustments and exit to see if the system passes the POST check.

Task 3: Cisco Packet Tracer LAN Topology

To connect 5 systems and 1 printer using a standard Class C subnet, we will use the 192.168.1.0 /24 private network space.

Network Architecture Details:

- Central Device: 1x Cisco Catalyst 2960 24-Port Switch.

- End Devices: 5x Generic Desktop PCs, 1x Network Printer.

- Media Cables: Straight-Through Copper Ethernet cables linking the FastEthernet network ports of all machines to the Switch interfaces.

IP Addressing Table Allocation:

| Device Name | Interface | IPv4 Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC-01 | FastEthernet0 | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 |
| PC-02 | FastEthernet0 | 192.168.1.12 | 255.255.255.0 | 192.168.1.1 |
| PC-03 | FastEthernet0 | 192.168.1.13 | 255.255.255.0 | 192.168.1.1 |
| PC-04 | FastEthernet0 | 192.168.1.14 | 255.255.255.0 | 192.168.1.1 |
| PC-05 | FastEthernet0 | 192.168.1.15 | 255.255.255.0 | 192.168.1.1 |
| Printer-01 | FastEthernet0 | 192.168.1.50 | 255.255.255.0 | 192.168.1.1 |

Task 4: Clean OS Installation & Disk Partitioning

- Insert the bootable OS installation flash drive, access the boot menu via F12, and boot into the setup installer environment.

- When prompted for installation type, choose Custom / Advanced Installation.

- Manual Partitioning Scheme: Delete all older raw allocations until you have clean unallocated disk space.

  - Create a 500 MB Boot/EFI System Partition (Formatted as FAT32 for bootloader files).

  - Create a Root/System Partition (C: Drive) allocating the majority space, formatted as NTFS (for Windows) or ext4 (for Linux).

  - (Optional for Linux): Create a dedicated Swap partition corresponding to the system's memory capacity.

- Proceed with installation execution. Once the desktop environment finishes loading, initialize Startup Optimization:

  - Open Task Manager (Ctrl + Shift + Esc) and go to the Startup Apps tab.

  - Right-click and Disable non-essential applications (like Cortana, OneDrive, or third-party launchers) to accelerate boot performance.

Section C: Integrated Case Study (Nexus Innovations)

1. Hardware Components Causing Random Reboots Under Load

When systems abruptly reboot out of nowhere without throwing a Blue Screen (BSOD) during heavy processing tasks, the three prime culprits are:

- CPU (Processor): Reaches its maximum thermal safety threshold junctions and triggers a hard shutdown to avoid melting.

- GPU (Graphics Card): Drawing immense power during 3D scene rendering workloads, generating rapid heat spikes.

- PSU (Power Supply Unit): Failing to supply stable clean wattage lines or tripping its internal Over-Temperature Protection (OTP) / Over-Current Protection (OCP).

2. Why They Pass POST in the Morning but Fail in the Afternoon

- The Explanation: In the morning, the workstations have been turned off all night. The ambient temperature inside the small unventilated desk cabinets is cool. When turned on, components pass the brief POST (Power-On Self-Test) because the system is running at idle temps with zero computational load.

- By afternoon, heavy rendering tasks stress the CPU/GPU constantly. The trapped, hot exhaust air has nowhere to escape inside the tight cabinet space, causing the ambient enclosure temperature to steadily climb. The cooling fans end up recycling trapped hot air, triggering thermal limits that force a protective emergency reboot.

3. Using Stress Test Tools to Confirm the Issue

To diagnose whether it is an electrical power failure or a thermal overheating issue:

- Download and run a combined hardware verification utility like AIDA64, Prime95 (for CPU), or FurMark (for GPU) while running hardware monitoring software like HWMonitor or HWInfo64.

- Watch the Diagnostics closely: * If the core temperatures climb continuously toward 95°C–100°C right before the machine cuts out, it confirms a Thermal Throttling / Cooling Failure.

  - If the system reboots instantly the exact moment the rendering stress test starts (while temperatures are still low), it indicates a Power Supply failure where the PSU cannot handle the sudden electrical current draw.

4. Three Remediation Steps

- Improve Ventilation (Immediate Fix): Relocate the computer towers completely out of the unventilated desk cabinets onto an open desktop surface or cut out large exhaust ventilation holes in the back of the wooden cabinets to install active exhaust mesh fans.

- Maintenance Cleaning: Open the case panels and use compressed air to thoroughly clean out accumulated dust clogs from the CPU heatsink fins, GPU fans, and case intake filters.

- Upgrade Case Fans: Adjust the internal chassis configuration to a Positive Airflow Setup by installing high-CFM static pressure intake fans at the front and exhaust fans at the top/rear of the computer chassis.

5. Technical Justification for a PSU Upgrade

- Why it might be necessary: Rendering software stresses both the CPU and GPU to maximum capacities at the exact same time. If the workstations were built using standard, generic office power supplies, the total power draw can exceed the PSU’s continuous rated wattage output capacity, causing the 12V voltage rail to sag and crash the system.

- Conclusion: If the diagnostic testing shows that temperatures are under control (below 80°C) but the systems still crash under load, the existing PSUs are inadequate. You must upgrade them to enterprise-grade 80-Plus Gold Certified PSUs with a higher power headroom rating (e.g., 750W or higher) to guarantee clean power delivery